Guest Services

Configuring Guest Services on SonicWall ***DRAFT ***

Configuring Guest Services on SonicWall Firewall Appliances

This document describes how to set up Guest Services on SonicWall Firewall Appliances.

What are Guest Services?

Many customer use SonicWall technology to secure their own internet access and protect the internal network. In some cases Wireless Technology from SonicWall is used. Now there are very often requests from customer & partner to get temporarily access to the internet. This applies to companies as well to Hotels and other public institutions. To avoid giving the guest full access to the internal network, SonicWall provides Guest Services on all devices with integrated wireless or devices that are connected to SonicPoints

With Guest Services temporary user accounts can be created and tracked. Those guest-accounts can be used on wireless and wired network-segments, and the user can be restricted to get only access to the internet, no local ressources are disclosed.

This document focuses on the pure guest service functionality – there are other documents available on how to set up and secure (wireless-) networks.

The original idea was to allow guest services only to clients connected wireless, but there are some cases where wired network - structure is already in place and where customer requested to allow guest services in specific segments on their network. That can be accomplishe as well, as long as this network is separated from the LAN (in fact, it has to be declared as Wireless, even if it does not use Wireless Technology

Prerequisits

Before starting to work with this document, we assume that you set up a Sonicwall device and have either integrated wireless or SonicPoints configured in a way to send out a unecrypted SSID. If you want to use Guest Services for wired users, one interface has to be defined as WLAN Interface and all connection points that should be able to use GS should be connected to this interface.

Configuring the Wireless Zone

Edit the WLAN-Zone on Interfaces / Zones.

First check the General Tab, it is recommended to switch off “Enable Client AV Enforcement Service” – otherwise any Guest will have to install a copy of Client AV. This setting applies to all users in this zone. It can be disabled only for guest users on the “Guest Services” Tab.

On the Wireless-Tab, disable the Checkbox “Only allow traffic generated by a SonicPoint” if you want to allow Guest from that zone to use a network cable (that has to be connected to the Wireless Interface). As well, to make access to guest services as easy as possible, it is recommended to switch off WiFi Sec enforcement. Be aware that everybody in this zone surfs in clear text, without encryption. If security matters in this zone, WiFi Sec can be enabled, but it demand futher configuration on the client. It will not work for devices connected via network-cable.

Guest Services Tab

Enable Wireless Guest Services by checking the corresponding checkbox.

There are a couple of options

- Enable inter-guest communication
Allows guests to communicate directly with each other

- Bypass AV-Check for Guests
If Client AV is enabled on this Zone, it will be bypassed for Guests. Enabling this prevents guests from installing the SonicWall AV Client. They are still protected by Gateway AV, if installed and enabled on the “General Tab” of the Wireless Zone.

- Enable Dynamic Address Translation
Grants access to non-DHCP guests. Even if the guest has configured any fixed IP on his WLAN Adapter on the computer, he will be able to use this service.

- Enable External Guest Authentication
This is briefly described in “Leightweight Hotspot Messaging” at the end of the document.

- Custom Authentication Page
This is used to easily create a landing page for the guest. Simple text can be used or it can be redirected to a URL, containing Test & Graphics for the Header & Footer of this page. When graphis are used, they can be stored on a Webserver – make sure the sonicwall has access to that server

- Post Authentication Page
This is the Website displayed after the user has successfully logged in. If nothing is specified, the website originally requested by the user will be displayed.

- Bypass Guest Authentication
Gives the ability to exclude specific MAC-Addresses from authenticating

- Redirect SMTP Traffic
Redirects SMTP traffic from this Zone to a SMTP Server.

Picture 1: Custom Login Page Settings

Deny Networks
Blocks traffic to the specified denied networks

- Pass Networks
Automatically allows traffic to the networks specified here

- Max Guests
Limits the number of maximum allowed guests

With these Settings, a guest should be able to connect to a Wireless Signal and get an IP-Address. As soon as the guest tries to access any Web-Page, he will be redirected to the Authentication Page, where he has to enter a username and password.

Those accounts can be created manually on the Firewall or can come from a directory – as configured under Users / Settings or /Local Users. The easiest way to create Guest User Account is to use the functionality “Guest Services”, to be found in the Users Section as well.

Here various profiles can be created, which can have different parameters and which allow the administrator to bulk-create accounts with specific properties.

Users / Guest Services

Under Users / Guest Services Profiles can be created. Here the Administrator can decide if the user should see a status window, which gives him important information about his Guest Status and where he has the option to terminate his session.

Guest Profiles

The “Add...” Button opens a new dialog to create a new profile, which can be used to create Guest User Accounts. There are a couple of options:

- Profile Name
Makes it easy to manage different profiles

- User Name Prefix
The system will automatically create Usernames, they will all start with the specified Prefix

- Auto-Generate User Names
The system will create random Usernames

- Auto-Generate Password
the system will create random passwords

- Enable Account
If checked, the account will be enabled after creation and ready to be used

- Auto Prune Account
The system will automatically delete an account when the Account Lifetime is expired

- Enforce login uniqueness
Each Username can only be used on one computer at the same time. Prevents multiple users from using the same guest account. Disabling can be helpful in an environment, where a group of people may share a single account (in a conference etc)

- Activate account upon first login
If this checkbox is checked, the account expiration begins, when the user logs in the first time. If it is not checked, the expiration begins immediately after the creation of the account.

- Account Lifetime
The time till the account expires. This depends on the setting  “Activate account upon first login “

- Session Lifetime
Is the time the Guest effectively can use internet-access

- Idle Timeout
The Guest Account usage will automatically terminated, if the user does not use it for the specified time. If the status window is displayed (as described before) the user can disconnect at any time.

- Comment
Here a comment for this profile can be added

How to use the various options

Example 1

When a guest account is configured with the options shown in the picture on the right, the following rules will apply: The Guest Account will start to expire after the User has logged in (Activate account upon first login). It will be automatically disabled and deleted after 7 days – regardless of how much time is left on the account. This prevents of having multiple partly used Guest Accounts within the system. If it is for example a business hotel, where people typically stay 1 or 2 nights, this is a good setting. During these 7 days, the user can connect and disconnect as often as he wants and spit his effective internet-time of 8 hours in as many small sessions as he wants (Example: connect, synchronize mails, disconnect)

If he should forget to disconnect (and his PC is not sending any information to the internet), he will be disconnected automatically after 10 minutes.

Example 2

This shows a profile to create “time-bombed” accounts. Here the user has 4 hours online time, but he/she cannot split the time, as Account Lifetime equals Session Lifetime. So as soon as the user logs in, he can use the internet for 4 hrs. Even if he/she logs out – the account will expire 4 hrs after log in and will be deleted.

Generating Guest User Accounts

Guest Accounts can be created under Users / Guest Accounts. They can be created manually one by one by clicking the “Add Guest…” Button, which will bring up the following dialog:

A big number of accounts can be created automatically by clicking the “Generate..” Button. This brings up this dialog:

Values like “User Name Prefix” and settings on the Guest Services Tab will be used as specified in the selected profile

After entering the number of accounts to be created, and clicking “OK”, the accounts will be created automatically and show up in the Guest Accounts Screen:

These can now be exported in a csv file by using the Export… button on the bottom of the screen. This file can be viewed in Excel / Word:

As well, vouchers can be created with a TextProcessor, a template is available for download as well:

User experience

A user that connects to the Wireless-Signal will get an IP-Address. As soon as he / she tries to access any page on the internet, he / she will be redirected to the authentication page of the Sonicwall

When the user enters correct login-credentials, the status window will pop-up and he will be redirected to the page he tried to acces (if configured on the Zone / Guest Services Tab)

Tracking Guest Users

In the SonicWall Admin-Menu under Users / Guest Status you can check the status of all currently active guest sessions

Integration into ViewPoint / Global Management System

ViewPoint shows details about the activities of Guest Users.

Managing Guest Services

In most cases, Guest Services will work for administrative employees without access to the Firewall administration. Just make sure there are enough valid vouchers on stock the employee (receptionist etc) can sell.

If there is a requirement to give an employee limited access to the firewall this can be done via the Group “Limited Administrator”

Create a Local User and add it to the group “Limited Administrators”

This user will have a limited access to the Management GUI. Wireless Guest Services (Creating / tracking / deleting user) is possible.

Lightweight Hotspot Messaging – Protocol

Lightweight Hotspot Messaging (LHM) leverages the SonicWALL Guest Service model, wherein users can

be classified and authorized for differentiated network access through a SonicWALL security appliance. For example, the

SonicWALL can be configured such that any user connecting through an interface belonging to a guest-services enabled

WLAN (wireless LAN) Zone will only have access to the Internet (Untrusted network), but will not have access to the LAN

(Trusted network). This allows a single network appliance to offer simultaneous access to trusted and guest users.

LHM extends the Guest Services model by breaking apart the authentication and authorization processes, allowing the authentication to occur external to the SonicWALL. This allows for extensive customization of the authentication interface, and also allows for any kind of imaginable authentication scheme to be used.

Martin Schmitz, Systems Engineer Germany July 2009